M&V Report

City of Northbridge

“Municipal Public Lighting System”

Annual Report for Determining Achieved Savings

2nd year of the guarantee period

Place: Northbridge

Date: 15.03.2026.

Report number: MV-ANON-2026-02

Anonymization and Legal Notice. This anonymized sample document has been prepared for review, presentation, and demonstration purposes only. Names of municipalities, companies, projects, contract references, dates, locations, numerical values, contact details, logos, and other identifying information have been replaced, generalized, or fictionalized. Any resemblance to an actual legal entity, public authority, private partner, contract, project, person, address, registration number, or financial result is coincidental and should not be interpreted as identifying a real party or disclosing confidential information. This document does not create legal rights or obligations, does not amend or replace any executed agreement, and should not be relied upon as legal, financial, technical, tax, accounting, or regulatory advice. The original rights, obligations, confidentiality protections, intellectual-property rights, and data-protection requirements of any underlying source material remain unaffected. Distribution should be limited to authorized recipients and handled in accordance with applicable confidentiality, data-protection, and document-control requirements.

# Executive Summary

This anonymized report presents the achieved energy, cost, maintenance, and environmental performance of the Municipal Public Lighting System during the 2nd year of the guarantee period. Based on corrected electricity consumption and reference operating-cost assumptions, the project achieved savings above the guaranteed level, supporting the Private Partner’s entitlement to the annual base fee and an additional fee under the agreement.

Contents

​

Executive Summary	2

1. Introduction	2

2. General Data	3

3. Guaranteed Consumption	3

4. Measured Consumption	4

5. Comparison of Operating Costs	4

6. Impact of Electricity Price Increase	5

7. Measures Applied for Preventive, Operational and Emergency Maintenance	6

9. Final Provisions	7

​

# 1. Introduction

This report has been prepared as a task and obligation of the Private Partner, as prescribed by the public-private partnership agreement “Public Lighting Energy Efficiency Project” No. PPP-ANON-2024-17 dated 12.04.2024.

The report presents data obtained during the 2nd year of the Guarantee Period. It has been prepared for monitoring savings, supervising the operation of the public lighting system by the Private Partner, and informing the Public Partner about the functioning of public lighting. The report has been prepared in accordance with the Measurement & Verification Plan and serves as the basis for calculating the annual fee and any additional fee or penalties related to achieved savings.

# 2. General Data

| Accounting period: |  | 2nd year of the guarantee period (from 01.01.2025 to 31.12.2025) |
| --- | --- | --- |
| Guarantee period: |  | 10 years |
| Start of guarantee period: |  | 01.01.2024 |
| Number of luminaires: |  | 3,248 pcs |
| Contract value: |  | EUR 1,840,000.00 |
| Guaranteed operating cost savings: |  | EUR 214,500.00/year |
| Guaranteed system consumption: |  | 1,245,600 kWh/year |
| Annual base fee: |  | EUR 171,600.00/year |
| Maintenance fee: |  | EUR 42,900.00/year |
| Reference electricity price: |  | 0.1416 EUR/kWh |
| Exchange rate on publication date: |  | 1 EUR = 117.30 RSD |
| System data before modernization (from the bid form): |  |  |
| Installed power of the public lighting system: |  | 612.4 kW |
| Reference electricity consumption: |  | 2,760,000 kWh/year |
| Reference system consumption: |  | 2,690,500 kWh/year |
| Reference maintenance costs: |  | EUR 88,400.00/year |
| Total reference operating costs: |  | EUR 469,900.00/year |
| System data after modernization (from the handover record): |  |  |
| Installed power without dimming: |  | 302.8 kW |
| Installed power with dimming: |  | 221.6 kW |
| Installed power of other consumers: |  | 18.5 kW |
| Projected public lighting system consumption: |  | 1,245,600 kWh/year |
| System data in the 2nd year of the guarantee period: |  |  |
| Electricity consumption of the entire system: |  | 1,318,420 kWh |
| Corrected electricity consumption: |  | 1,238,900 kWh |
| Corrected consumption - funds spent using the reference energy price: |  | EUR 175,446.24 |

*No new handover records or system changes were recorded during the reporting period.

# 3. Guaranteed Consumption

The guaranteed consumption of the public lighting system, based on bid No. BID-ANON-2024-09 dated 15.02.2024, amounts to 1,245,600 kWh/year.

This value applies to the part of the public lighting system scheduled for replacement.

Other consumers were not included in the tender documentation. They were identified in the handover record, and their consumption should therefore be deducted from actual consumption.

# 4. Measured Consumption

The following table presents measured monthly consumption. The reference electricity price shown in Section 2 – General Data of this report was used to determine the funds spent. The guaranteed consumption stated in the bid was calculated for the part of the system covering only the replaced luminaires. Therefore, corrected consumption is calculated based on actual consumption by deducting additional consumers and adding the consumption of remaining luminaires. Based on this corrected consumption, electricity costs for the relevant accounting period of the Guarantee Period can be calculated.

Table – Electricity consumption of the public lighting system in the 2nd year of the guarantee period

| Year | Month | Electricity consumed according to invoices [kWh] | Corrected electricity consumption according to invoices [kWh] | Corrected electricity consumption - funds spent excluding VAT at reference energy price [EUR] |
| --- | --- | --- | --- | --- |
| 2025 | January | 124,850 | 117,500 | 16,638.00 |
| 2025 | February | 113,420 | 106,800 | 15,122.88 |
| 2025 | March | 108,760 | 102,100 | 14,457.36 |
| 2025 | April | 94,260 | 88,700 | 12,559.92 |
| 2025 | May | 82,540 | 77,500 | 10,974.00 |
| 2025 | June | 73,120 | 68,900 | 9,756.24 |
| 2025 | July | 69,880 | 65,600 | 9,288.96 |
| 2025 | August | 74,230 | 69,800 | 9,883.68 |
| 2025 | September | 89,710 | 84,200 | 11,922.72 |
| 2025 | October | 115,360 | 108,000 | 15,292.80 |
| 2025 | November | 172,650 | 163,400 | 23,137.44 |
| 2025 | December | 199,640 | 186,400 | 26,394.24 |
|  | Σ | 1,318,420 | 1,238,900 | 175,446.24 |

Electricity costs for the 2nd year of the Guarantee Period, calculated using the reference electricity price and the corrected consumption total of 1,238,900 kWh, amount to EUR 175,446.24.

# 5. Comparison of Operating Costs

According to the agreement, the costs for operational and preventive maintenance of the public lighting system in the City of Northbridge amount to EUR 42,900.00/year. Based on the previous table, operating costs in the 2nd year of the Guarantee Period amount to EUR 175,446.24.

Total operating costs in the 2nd year amount to EUR 175,446.24 + EUR 42,900.00 = EUR 218,346.24.

The achieved savings in operating costs represent the difference between the reference operating costs and the actual operating costs, as follows:

EUR 469,900.00 - EUR 218,346.24 = EUR 251,553.76.

The percentage ratio between the achieved savings difference and the guaranteed operating-cost savings is 117.27%.

The difference indicates that the Private Partner achieved savings 17.27% higher than the guaranteed savings. Since the value exceeds the +/- 5% threshold, the Private Partner has acquired the right to an additional fee according to the agreement.

By fulfilling its contractual obligation, the Private Partner acquired the right to the annual base fee, which under the agreement for the first 10 years of the guarantee period amounts to 80% of the guaranteed financial savings in operating costs, i.e. EUR 214,500.00 × 80% = EUR 171,600.00.

# 6. Impact of Electricity Price Increase

Based on the bid form, the reference annual electricity costs of the public lighting system of the City of Northbridge amounted to EUR 381,500.00 at the reference electricity price of 16.61 RSD/kWh, using the middle exchange rate on the date of publication of the public invitation. Reference maintenance costs amounted to EUR 88,400.00.

Reference system consumption amounted to 2,690,500 kWh.

In the 2nd year of the guarantee period, the electricity price for the public lighting category increased by 12%, amounting to 18.60 RSD/kWh (including all charges, excluding VAT).

If the City had not implemented the public-private partnership project for replacing old luminaires with new LED luminaires, the current costs of the City of Northbridge would be:

2,690,500 kWh × 18.60 RSD = 50,043,300.00 RSD, which at the middle exchange rate on the date of publication of the public invitation amounts to EUR 426,626.60.

The following chart illustrates the difference between the estimated operating costs without replacement and the operating costs after replacement in the 2nd year of the guarantee period. Where current electricity prices are used, they are applied for illustrative comparison only and do not change the reference-price methodology used for the guaranteed-savings calculation.

*All stated amounts exclude VAT and include additional consumers.

After implementation of the public lighting project, electricity costs decreased significantly and, for the 2nd year of the guarantee period, amount to EUR 175,446.24 at the current electricity price, representing savings of 58.88% compared with the reference system costs.

# 7. Measures Applied for Preventive, Operational and Emergency Maintenance

For the accounting period from 01.01.2025 to 31.12.2025, quarterly reports were submitted for each three-month period separately, containing all measures applied in relation to preventive, operational, and emergency maintenance.

Based on those reports, it can be concluded that the Private Partner fulfilled all obligations related to maintenance of the public lighting system within the deadlines prescribed by the M&V Plan, while following the procedural steps agreed with the Public Partner.

The quarterly reports show that the Private Partner responded to all reported failures within 48 hours after receiving notification and remedied them as soon as possible, where the failure was within its responsibility.

The highest number of faulty luminaires at the same time in the 2nd year of the Guarantee Period was 14 units, meaning that more than 99.5% of the system operated properly at the same time.

8. Environmental Protection Measures

CO2 emissions in the 2nd year of the guarantee period

For anonymized reporting purposes, the calculation applies a sample emission factor of 0.00053 t CO2/kWh for electricity savings. This factor is used only for the illustrative calculation in this anonymized document.

Reference consumption of the old public lighting system amounted to 2,690,500 kWh annually.

Actual consumption of the new system, based on electricity invoices, in the 2nd year of the guarantee period amounted to 1,318,420 kWh.

Achieved electricity savings amount to: 2,690,500 kWh - 1,318,420 kWh = 1,372,080 kWh

By replacing the public lighting luminaires with new LED luminaires in the 2nd year of the guarantee period, the Municipality/City of Northbridge reduced CO2 emissions by:

1,372,080 kWh × 0.00053 t/kWh = 727.20 t

# 9. Final Provisions

The contracting parties agree with the content of this report and accept the measures applied during the 2nd year of the Guarantee Period.

The contracting parties agree that the Private Partner fulfilled its contractual obligations during the 2nd year of the Guarantee Period.

The contracting parties agree that the Private Partner has acquired the right to the annual base fee.

The contracting parties agree that the Private Partner has acquired the right to an additional fee and is not obliged to pay penalties.

The report was prepared by “Anonymized ESCO Partner LLC”, as the lead contractor and party responsible for the functioning and maintenance of the active system elements.

If the Public Partner submits no objections within 15 days after receiving this report, the report shall be deemed approved by the Public Partner.

Recipients:

Public Partner

Private Partner

Prepared in two identical copies.

Approval and Signature Section

| For the Public Partner Name and title: ______________________________ Signature: ___________________________________ Date: _______________________________________ |  | For the Private Partner Name and title: ______________________________ Signature: ___________________________________ Date: _______________________________________ |
| --- | --- | --- |